ESP APM — ENGINEERING SERVICE

DEDICATED GRANULAR ARCHITECTURE + IMPLEMENTATION DESIGN

Grounded in the supplied ESP Engineering Research Package, engineering_calculation_registry.yaml, engineering_input_schema.json, engineering_reference_registry.yaml, and engineering_test_cases.yaml. This document defines the deterministic Engineering Service boundary, data flow, calculation execution model, validation gates, API/tool exposure, testing, and production implementation sequence.

# 1. Purpose and Design Decision

The Engineering Service is the deterministic engineering-calculation boundary of the ESP Agentic platform. It computes approved calculations from authoritative inputs; the LLM may request/explain results but must not generate or silently modify formulas, constants, limits, or engineering assumptions.

This directly follows the research-package governing rule: the LLM is not the calculation authority, and formulas, correction factors, limits and constants must be fundamental, explicitly sourced, or marked REQUIRES ENGINEERING VALIDATION. [ESP_Engineering_Research_Package.md, §Purpose + Governing rule]

# 2. Architecture Position in the Overall Agentic System

USER / EVENT
    |
OBJECTIVE
    |
LANGGRAPH SUPERVISOR
    |
SPECIALIST
    |
ENGINEERING TOOL
    |
POLICY / ACL
    |
ENGINEERING API
    |
ENGINEERING ORCHESTRATOR
    |
+-------------------------------+
| Input Validation              |
| Reference / Config Resolution |
| Calculation Registry          |
| Deterministic Engine         |
| Result Validation             |
| Provenance / Audit            |
+-------------------------------+
    |
STRUCTURED ENGINEERING RESULT
    |
EVIDENCE PACK
    |
AGENT / XAI / UI

The service therefore sits behind the Tool Registry and PolicyEngine and in front of deterministic calculation functions.

# 3. Authoritative Design Artifacts

| Artifact | Role in the Engineering Service | Grounded fact |
| --- | --- | --- |
| engineering_calculation_registry.yaml | Executable calculation catalogue/configuration | Defines IDs, formulas, inputs, validity, outputs, dependencies and statuses; service must not regenerate formulas at runtime. |
| engineering_input_schema.json | Canonical input envelope/schema | Defines asset, runtime, fluid/PVT, well, ESP, OEM, client-limit and reliability data. |
| engineering_reference_registry.yaml | Reference/authority catalogue | Tracks standards, editions/status, applicability and validation gaps. |
| engineering_test_cases.yaml | Verification and fail-closed tests | Separates synthetic arithmetic checks from real-data placeholders and failure-mode checks. |
| ESP_Engineering_Research_Package.md | Research basis and data requirements | Defines calculation intent, input dependencies, standards/OEM boundaries and unresolved validation items. |

# 4. Core Architectural Principles

Deterministic execution: identical validated inputs + versioned configuration produce reproducible outputs.

Fail-closed: missing required inputs, unapproved limits or unresolved engineering validation block the dependent calculation rather than using a generic default.

Reference-aware: every material result carries calculation/reference/version/provenance metadata.

Asset-aware: calculations use the exact installed equipment and asset-specific data where required.

No LLM calculation: the LLM calls typed engineering tools and explains the returned result.

No direct source access from the Agent: source data is retrieved through controlled adapters/services.

Explicit uncertainty: approximations, correlation choices, curve interpolation, or unresolved applicability remain visible.

No universal defaults: especially for OEM curves, ROR, vibration, motor temperature, underload/overload thresholds and PVT correlations.

# 5. Service Internal Components

| Component | Responsibility | Input | Output |
| --- | --- | --- | --- |
| API layer | Accept typed calculation requests | CalculationRequest | CalculationResponse |
| Request validator | Validate schema + field types | Request | Validated request / errors |
| Reference resolver | Resolve calculation/reference/OEM/client configuration | Calc ID + asset | Reference/config set |
| Calculation registry loader | Load immutable runtime config | Registry | Calculation definition |
| Input resolver | Join asset/runtime/OEM/client/PVT inputs | Validated request | Resolved calculation inputs |
| Deterministic engine | Execute approved formula/lookup/statistics | Resolved inputs | Raw calculation result |
| Result validator | Validate range/unit/result semantics | Raw result | Validated result |
| Provenance builder | Attach sources, versions, timestamps | Result + inputs | Provenance |
| Audit publisher | Record run/calculation trace | Result metadata | Audit record |
| Tool adapter | Expose approved calculations to Agent | Tool request | Structured result |

# 6. Runtime Execution Pipeline

1. Receive calculation request
2. Validate request envelope
3. Validate calculation ID
4. Load calculation definition
5. Check calculation status
6. Resolve asset context
7. Resolve required runtime data
8. Resolve OEM/reference/client configuration
9. Validate required inputs + units
10. Validate operating/validity conditions
11. Execute deterministic calculation
12. Validate result
13. Attach provenance + version
14. Persist/audit
15. Return structured result

# 7. Calculation State Machine

REQUESTED
   |
VALIDATING
   |
+--+------------------------+
|                           |
VALID                       INVALID
|                           |
CHECK AUTHORITY              REJECT
|
CHECK INPUTS
|
+-------------+-------------+
|                           |
READY                    BLOCKED
|                        (missing/validation)
EXECUTE                      |
|                            |
VALIDATE RESULT <------------+
|
PERSIST / AUDIT
|
COMPLETED

# 8. Calculation Registry Semantics

The calculation registry is the central execution configuration. Its header explicitly states that the service must treat it as authoritative configuration and must not regenerate formulas at runtime. Calculations marked requires_validation must not execute until the flagged issue is resolved. [engineering_calculation_registry.yaml, lines 9–13]

| Registry field | Meaning |
| --- | --- |
| id | Stable calculation identifier |
| name | Human-readable calculation |
| domain | Engineering domain |
| formula | Deterministic implementation definition |
| required_inputs | Mandatory runtime/asset/reference fields |
| optional_inputs | Optional refinements |
| units | Input/output units |
| validity_conditions | Preconditions |
| operating_range | Valid domain |
| output | Named result fields |
| reference | Authority basis |
| OEM dependency | Manufacturer-specific dependency |
| asset dependency | Asset-specific dependency |
| failure_condition | Fail-closed behavior |
| status | ready / requires_oem_data / requires_validation / requires_configuration |

# 9. Calculation Families

| Family | Representative calculations | Implementation posture |
| --- | --- | --- |
| A — Fundamental / physics | A1 Head↔Pressure, A2 Hydraulic Power, A4 Affinity Laws, A5 %BEP | Start here; deterministic math, subject to input validity. |
| B — OEM data dependent | B1 Pump Curve, B2 Motor Lookup, B3 Cable Voltage Drop | Implement framework + real OEM datasets; block until matching data is loaded. |
| C — Asset dependent | C1 System Operating Point, C2 Motor Load % | C2 is straightforward; C1 needs client/engineering-selected IPR and well/PVT inputs. |
| D — Fluid/PVT | D1 Viscosity, D2 Free Gas, D3 Gas Derating, D4 Mixture SG | D4 can start; D1–D3 have validation/configuration dependencies. |
| E — Client/OEM limits | E1 ROR, E2 Vibration, E3 Motor Temperature | Framework only until approved numeric limits are configured. |
| F — Diagnostic features | F1 Current Trend, F2 Cyclic Detection, F3 Load Flag | Statistical/config-driven; not universal standards. |
| G — Reliability | G1 Failure Classification, G2 MTBF, G3 MTTR | Useful later; G1 requires teardown taxonomy/report structure. |

# 10. Recommended MVP Calculation Set

For the first Agentic demo, do not wait for the complete engineering catalogue. Implement a small, defensible set:

| Priority | Calculation | Why it matters |
| --- | --- | --- |
| P0 | A1 — Pressure ↔ Head | Converts field pressure difference to pump-curve head. |
| P0 | A2 — Hydraulic Power | Converts flow/head/SG into deterministic hydraulic power. |
| P0 | A4 — Frequency/Speed Correction | Allows base OEM curve values to be related to operating frequency. |
| P0 | A5 — %BEP | Provides a simple operating-point health/performance metric. |
| P0 | C2 — Motor Load % | Uses measured current against exact installed motor rating. |
| P0 | F1 — Current Trend Slope | Adds objective trend evidence without inventing physical models. |
| P1 | B1 — Pump Curve Lookup | High-value but blocked until exact OEM curve is loaded. |
| P1 | E1 — ROR Check | High-value for applicability/operating-envelope questions once approved limits exist. |

The registry already marks A1, A2, A4, A5, C2 and G2 as ready, while B1/B2/B3 and several E/D/C calculations require OEM/client/validation inputs. [engineering_calculation_registry.yaml]

# 11. Input Architecture

Calculation Request
   |
   +--> asset_identity
   |
   +--> real_time_or_periodic_data
   |
   +--> fluid_data
   |
   +--> well_reservoir_data
   |
   +--> esp_equipment_data
   |
   +--> oem_reference_data
   |
   +--> client_approved_limits
   |
   +--> reliability_data

These sections are defined in the supplied JSON Schema. [engineering_input_schema.json]

# 12. Input Authority Matrix

| Data domain | Typical examples | Authoritative source in supplied schema |
| --- | --- | --- |
| Asset identity | asset_id, well_name, field | Asset/registry |
| Runtime | ΔP, Q, frequency, current, vibration, temperature | SCADA / VSD / sensors |
| Fluid | water cut, oil gravity, water SG, viscosity, PVT source | Test separator / lab / PVT |
| Well/reservoir | reservoir pressure, IPR selection/params, tubing ID | Well test / engineering |
| ESP equipment | pump model, stage count, motor model, cable, depth | Installed equipment record |
| OEM reference | pump curve, motor nameplate, cable impedance, gas curve | OEM master catalog/datasheet |
| Client limits | ROR, vibration, temperature, load thresholds | Client-approved policy / OEM / approved engineering |
| Reliability | teardown records, failure categories, dates | Work-order / teardown / reliability system |

# 13. Real-Time Data Mapping for Current Asset Dataset

| Existing telemetry/asset input | Engineering field | Use |
| --- | --- | --- |
| Liquid rate | flow_rate_q_bpd | A2, A5, C1, E1 |
| Intake pressure + discharge pressure | flowing_pressure_diff_psi | A1; service may derive ΔP from raw PIP/PDP upstream |
| Motor current | motor_measured_current_a | C2, F1, F3 |
| Frequency | operating_frequency_hz | A4, B1, B2 |
| Motor temperature | motor_temperature_measured | E3 |
| Vibration | vibration_measured | E2 |
| Water cut | water_cut | D4 |
| Oil gravity | oil_gravity_api | D4 |
| Produced-water SG | produced_water_sg | D4 |
| Pump/motor model | pump_model_id / motor_model_id | B1/B2/C2/A5 |
| Stage count | stage_count | B1/A3 |
| Setting depth | setting_depth_ft | A1/C1 |

The engineering schema explicitly maps many of these fields to calculation IDs; e.g. Q to A2/A5/C1/E1, frequency to A4/B1/B2, and measured current to C2/F1/F3. [engineering_input_schema.json]

# 14. OEM Master Data Layer

OEM data is not supplied as a free-text prompt to the Agent. It should be stored in structured, versioned master datasets keyed by the exact installed model/series/stage configuration.

OEM Master Data
├── pump_model_id
├── stage_count / stage type
├── base_frequency
├── pump_curve[]
│   ├── Q
│   ├── H
│   ├── efficiency
│   └── BHP
├── ROR bounds
├── motor_model_id
├── motor_nameplate
├── motor performance curves
├── cable impedance table
├── gas separator model
├── gas-handling curve
└── OEM limits

The input schema explicitly states OEM reference data is joined from an OEM master catalog keyed by pump_model_id/motor_model_id. [engineering_input_schema.json, lines 847–857]

# 15. Client-Approved Limits Layer

Limits must be external configuration, not constants embedded in code. The supplied schema explicitly says no service-side defaults are permitted for client-approved limits. [engineering_input_schema.json, lines 861–870]

Asset / Fleet Configuration
        |
        +--> ROR
        +--> vibration
        +--> motor temperature
        +--> underload
        +--> overload
        +--> cyclic threshold
        +--> approved gas-handling correlation

# 16. Reference Registry

The reference registry is a governance layer, not a formula engine.

| Reference state | Service behavior |
| --- | --- |
| Current + applicable | May be used when the calculation explicitly references it. |
| Requires engineering validation | Do not hard-code its unresolved content; dependent calculation remains blocked. |
| Withdrawn/superseded | Do not use as current authority. |
| Third-party verification only | Re-verify directly before compliance-critical production use. |
| Peer-reviewed correlation | Must be explicitly selected/configured; never silently default. |

The reference registry itself states that third-party verification should be rechecked directly against primary API/ISO/ANSI sources before compliance-critical use. [engineering_reference_registry.yaml, lines 416–420]

# 17. Important Reference Findings to Preserve

API RP 11S4 is marked WITHDRAWN in the supplied reference registry; it must not be treated as current authority.

ISO 15551:2023 is identified as current and as the important current framework for ESP systems and ROR methodology, but the numeric ROR annex content was not retrieved and remains a validation dependency.

ANSI/HI 9.6.7-2021 is identified as current for viscous-liquid performance correction, but ESP-specific applicability and exact chart content require direct verification before hard-coding.

API RP 11S8 numeric vibration limits were not retrieved; do not hard-code numeric limits from memory.

PVT correlations such as Standing/Vasquez-Beggs/Beggs-Brill/Turpin are described as literature/correlation choices requiring explicit client/engineering selection.

These points are directly reflected in the supplied reference registry and research package. [engineering_reference_registry.yaml, lines 465–474, 506–519, 532–545, 486–495, 595–605]

# 18. Deterministic Calculation Function Boundary

Calculation Registry
       |
       v
Typed Function
       |
       +--> validated scalar inputs
       +--> validated reference/config
       +--> no network calls
       +--> no LLM
       +--> no mutable global state
       |
       v
Typed CalculationResult

Keep the numerical functions pure wherever possible. All data retrieval belongs outside the calculation function.

# 19. Calculation Result Contract

CalculationResult
├── calculation_id
├── calculation_version
├── status
├── asset_id
├── executed_at
├── inputs[]
├── outputs[]
├── units
├── assumptions[]
├── limitations[]
├── validity[]
├── reference_refs[]
├── oem_data_refs[]
├── client_policy_refs[]
├── quality
├── uncertainty
└── correlation_id

# 20. Example: A1 Execution

Inputs:
ΔP = PDP - PIP
SG = flowing mixture SG

Validate:
SG > 0
same asset/time window

Execute:
H_ft = (ΔP_psi × 2.31) / SG

Return:
head_ft
formula_id = A1_HEAD_PRESSURE_CONVERSION
status = VALID
provenance = ...

A1 is explicitly defined as a fundamental fluid-statics conversion and is marked ready, while missing SG must fail closed. [engineering_calculation_registry.yaml, lines 18–34]

# 21. Example: B1 OEM Pump Curve

Input:
pump_model_id
stage_count
base_frequency
target_Q

Resolve:
exact OEM curve

Validate:
target_Q within published curve range

Interpolate:
H, efficiency, BHP

If no exact curve:
BLOCK
REQUIRES_OEM_DATA

Never:
substitute a "similar" pump curve silently

This fail-closed behavior is explicit in B1. [engineering_calculation_registry.yaml, lines 105–119]

# 22. Example: C1 System Operating Point

Reservoir pressure
+ IPR model + parameters
+ well geometry
+ PVT
+ pump curve
+ frequency
          |
          v
Iterative Q solution:
P_IPR(Q) = P_VLP(Q)
          |
          v
Q_operating_bpd
PIP_psi

C1 is deliberately marked as requiring client/engineering selection of the IPR and multiphase-flow correlation; the service must not default silently to a single model. [engineering_calculation_registry.yaml, lines 154–168]

# 23. Unit and Semantic Validation

Check every input unit before calculation.

Do not silently convert ambiguous sensor units.

Track per-stage versus total-stack quantities for pump data.

Track standard-condition versus flowing-condition volume semantics.

Ensure frequency/base-frequency pairs use a consistent basis.

Ensure current and rated current use the same phase/reference basis.

Attach the transformed/normalized input to the audit record.

# 24. Failure-Closed Rules

| Condition | Required behavior |
| --- | --- |
| Missing SG for A1 | Reject; no default SG=1.0 |
| Missing base frequency for A4 | Reject; do not assume 60 Hz |
| No OEM curve for B1 | Reject; no similar-pump substitution |
| No motor OEM data for B2 | Reject |
| No cable spec for B3 | Reject |
| No IPR selection for C1 | Reject / engineering review |
| No PVT model for D2 | Reject |
| No approved gas-handling model for D3 | Do not run |
| No ROR for E1 | Do not run |
| No vibration limit for E2 | Fail closed |
| No temperature limit for E3 | Fail closed |
| Sparse trend data for F1 | Return insufficient_data, not misleading slope |

These rules are directly reflected in the calculation registry and fail-closed test cases. [engineering_calculation_registry.yaml; engineering_test_cases.yaml]

# 25. Test Architecture

UNIT TESTS
  |
  +--> calculation math
  +--> validation
  +--> unit conversion
  +--> failure conditions
  |
INTEGRATION
  |
  +--> registry + input schema + reference resolver
  +--> OEM data lookup
  +--> Asset Context / Telemetry
  |
CONTRACT
  |
  +--> REST
  +--> Agent Tool
  |
E2E
  |
  +--> Agent → Engineering Tool → Service → Result → Evidence → UI

# 26. Existing Test Assets

The supplied test file already provides synthetic arithmetic checks for A1, A2, D4 and G2, plus explicit failure-mode checks and real-data placeholders.

| Test category | Examples | Meaning |
| --- | --- | --- |
| Synthetic arithmetic | A1, A2, D4, G2 | Tests deterministic arithmetic only; values are not field truth. |
| Failure-mode | Missing SG, missing base frequency, missing ROR, missing gas model | Tests fail-closed behavior. |
| Real-data placeholders | B1/B2/B3/C1/D1/D2/D3/E1/E2/E3/F1/F2/F3/G1/G3 | Defines exactly what real OEM/client/asset data is needed before numeric validation. |

The test file explicitly warns that synthetic cases are not real field/asset reference cases and that the placeholder cases must be populated from actual OEM/lab/client data. [engineering_test_cases.yaml, lines 615–630]

# 27. Real Engineering Validation Test Strategy

| Stage | Data required | Acceptance |
| --- | --- | --- |
| A calculations | Validated scalar test vectors | Numerical result within approved tolerance |
| B1 pump curve | Exact OEM digitized curve | Interpolation matches controlled reference points |
| B2 motor | Exact nameplate/curve | Lookup matches source |
| C1 operating point | Well/PVT/IPR + pump curve | Engineer-reviewed convergence + sensitivity |
| D1 viscosity | Purchased/validated HI chart + OEM base curve | Engineer-approved applicability |
| D2/D3 gas | Approved PVT + OEM/correlation | Engineer-approved validity range |
| E limits | Client/OEM/approved limits | Correct pass/fail at boundary cases |
| F diagnostics | Client-approved windows/thresholds | Stable, documented signal behavior |
| G reliability | Real fleet dates/teardown taxonomy | Population definition + data QA |

# 28. Service API Design

| Endpoint | Purpose |
| --- | --- |
| POST /api/v1/engineering/calculate | Execute one named calculation |
| POST /api/v1/engineering/batch | Execute a controlled set of calculations |
| GET /api/v1/engineering/calculations | List calculation definitions and status |
| GET /api/v1/engineering/calculations/{id} | Get calculation metadata/schema |
| GET /api/v1/engineering/references/{id} | Reference metadata |
| GET /api/v1/engineering/assets/{asset_id}/readiness | Show available inputs/data readiness |
| GET /api/v1/engineering/runs/{run_id} | Retrieve calculation audit/result |

# 29. Example Calculation Request

{
  "calculation_id": "A1_HEAD_PRESSURE_CONVERSION",
  "asset_id": "FS-031",
  "inputs": {
    "delta_p_psi": 1647.2,
    "fluid_sg": 0.90
  },
  "source_refs": {
    "telemetry_window": "...",
    "asset_context_version": "..."
  }
}

# 30. Example Calculation Response

{
  "calculation_id": "A1_HEAD_PRESSURE_CONVERSION",
  "calculation_version": "1.x",
  "status": "VALID",
  "asset_id": "FS-031",
  "outputs": {
    "head_ft": 4228.4
  },
  "units": {
    "head_ft": "ft"
  },
  "assumptions": [
    "single-phase liquid"
  ],
  "provenance": {
    "input_refs": [...],
    "reference_refs": [...]
  }
}

Illustrative payload shape only; the final API envelope should conform to the project's shared REST schema. The numerical example should not be treated as validated field truth.

# 31. Agent Tool Layer

LangGraph Specialist
        |
Engineering Tool
        |
Tool Registry
        |
PolicyEngine
        |
Engineering REST Service
        |
Deterministic calculation

| Tool | Initial exposure |
| --- | --- |
| calculate_head_from_delta_p | A1 |
| calculate_hydraulic_power | A2 |
| apply_frequency_correction | A4 |
| calculate_percent_bep | A5 |
| calculate_motor_load | C2 |
| calculate_current_trend | F1 |
| lookup_pump_curve | B1, after OEM data is loaded |
| check_ror | E1, after approved bounds exist |

# 32. REST + MCP Position

REST should remain the canonical Engineering Service interface. MCP is an optional Agent-facing interface/adapter when the capability is useful to expose through an MCP tool server. Both paths must call the same underlying Engineering Service/business logic.

Agent
 | | \ 
 |  +--> MCP Tool Adapter ----+
 |                           |
 +----> REST Tool Adapter ---+--> Engineering Service
                                |
                           Deterministic Engine

Do not implement separate calculation logic for REST and MCP.

# 33. Evidence Integration

Engineering Result
      |
EvidenceItem
├── calculation_id
├── result
├── units
├── inputs
├── assumptions
├── source refs
├── version
└── quality
      |
Evidence Pack
      |
Agent Context
      |
Advisory / XAI

This makes calculations visible in the same evidence chain as telemetry, ML, KB and historical cases.

# 34. XAI / UI Integration

| Engineering result | UI/XAI representation |
| --- | --- |
| A1 head | Formula/inputs + calculated head card |
| A4 frequency correction | Base vs adjusted performance visual |
| A5 %BEP | Operating point/BEP visual |
| C2 motor load | Current vs rated comparison |
| F1 trend | Trend line with slope |
| B1 pump curve | OEM pump curve + current operating point |
| E1 ROR | Operating point against approved envelope |

The Engineering Service should return structured data/specifications; the frontend renders charts/visual stories. The LLM explains the result but does not author the underlying calculation.

# 35. Data Readiness Endpoint

A valuable addition is a readiness endpoint so the Agent and UI can know what can be calculated before attempting execution.

GET /api/v1/engineering/assets/FS-031/readiness

returns:
- available calculations
- blocked calculations
- missing inputs
- missing OEM data
- missing client limits
- validation flags
- last source timestamps

This directly reflects the supplied placeholder/test architecture, where many calculations are pending real OEM/client/PVT inputs. [engineering_test_cases.yaml, lines 697–765]

# 36. Calculation Dependency Graph

D4 Fluid SG
     |
     +--> A1 Head
     |      |
     |      +--> A2 Hydraulic Power
     |               |
     |               +--> A3 BHP
     |
B1 Pump Curve ----> A5 %BEP
     |
     +-------------> C1 Operating Point
                           |
                           +--> E1 ROR
A4 Frequency Correction --+
B2 Motor Data -----------> C2 Motor Load
D2 Free Gas -------------> D3 Gas Derating
B1 Water Curve ----------> D1 Viscosity
F1/F2/F3 ----------------> diagnostic evidence

# 37. Implementation Directory

engineering_service/
├── api/
│   ├── routes.py
│   └── schemas.py
├── core/
│   ├── registry.py
│   ├── executor.py
│   ├── validation.py
│   └── provenance.py
├── calculations/
│   ├── section_a.py
│   ├── section_b.py
│   ├── section_c.py
│   ├── section_d.py
│   ├── section_e.py
│   ├── section_f.py
│   └── section_g.py
├── data/
│   ├── oem/
│   ├── client_limits/
│   └── references/
├── adapters/
│   ├── asset_context.py
│   ├── telemetry.py
│   ├── pvt.py
│   └── oem_catalog.py
├── tools/
│   └── engineering_tools.py
├── audit/
│   └── repository.py
└── tests/
    ├── unit/
    ├── integration/
    ├── contract/
    └── golden/

# 38. Implementation Sequence

| Sprint | Scope | Exit criteria |
| --- | --- | --- |
| ENG-1 | Service skeleton + contracts | API starts; shared schemas validate |
| ENG-2 | Registry loader + fail-closed executor | Registry is authoritative at runtime |
| ENG-3 | A1/A2/A4/A5 | Synthetic tests pass; deterministic outputs reproducible |
| ENG-4 | C2/F1 | Trend/load tests pass |
| ENG-5 | OEM master-data adapter + B1 | One exact real pump curve executes |
| ENG-6 | B2 + motor loading | Exact motor data integrated |
| ENG-7 | E1 limits/readiness | Approved ROR checks work |
| ENG-8 | Evidence/provenance | Result links to input/reference records |
| ENG-9 | Agent Tool + LangGraph | Specialist can request calculations |
| ENG-10 | REST + optional MCP | Equivalent result paths |
| ENG-11 | UI/XAI | Engineering results visualized in chat |
| ENG-12 | Validation/golden scenarios | Engineer-reviewed cases pass |

# 39. Phase Dependencies

| Dependency | Required capability |
| --- | --- |
| Asset Context | Exact asset equipment, hierarchy, installed pump/motor, mapping |
| Telemetry | Current values, timestamps, QoD |
| PVT/Fluid | Flowing SG, viscosity, water cut, oil gravity, PVT |
| OEM master data | Pump curves, motor nameplate/performance, cable/gas data |
| Client policy | ROR/temperature/vibration/load thresholds |
| Objective Layer | Which calculations are required for each workflow |
| Tool Registry | Agent-facing capability definitions |
| PolicyEngine | Authorization for calculation/tool use |
| Evidence Pack | Calculation provenance and claim trace |
| Audit Service | Reproducible run trace |

# 40. Production Hardening

Version calculation registry and reference registry together; record effective dates.

Version every OEM dataset and client-limit configuration.

Make calculation implementations immutable within a service version.

Do not allow ad-hoc formula edits through the UI.

Require review/approval workflow for reference or limit changes.

Monitor calculation failures, missing-input rates and blocked-calculation reasons.

Record exact input snapshot, source timestamps and calculation version for every material result.

Maintain rollback to prior calculation/service versions.

# 41. Security

| Boundary | Control |
| --- | --- |
| Agent → Tool | Objective/specialist ACL |
| Tool → Engineering API | Service identity + authorization |
| Engineering → Source adapters | Tenant/asset scope |
| OEM/client data | Controlled access; immutable versioned records |
| Reference changes | Privileged review |
| UI | Never trust UI authorization; enforce server-side |

# 42. Observability

| Metric | Purpose |
| --- | --- |
| calculation_requests_total | Usage |
| calculation_failures_total | Reliability |
| blocked_calculations_total | Data/config readiness |
| calculation_latency | Performance |
| oem_lookup_misses | Data coverage |
| client_limit_missing | Configuration gap |
| invalid_input_count | Data quality |
| result_validation_failures | Engine health |
| engineering_run_trace | Audit/reproducibility |

# 43. Acceptance Scenarios

| Scenario | Expected result |
| --- | --- |
| Current performance | A1/A2/A4/A5/C2 results available with provenance where inputs exist |
| Frequency change question | Engineering tool returns only validated/base-curve-derived outputs |
| No OEM curve | B1 blocks with REQUIRES_OEM_DATA |
| No approved ROR | E1 blocks with configuration reason |
| Missing SG | A1 fails closed |
| Stale/invalid telemetry | Validation rejects/qualifies input; no fabricated value |
| Agent asks for unsupported calculation | Tool registry/registry status prevents execution |
| UI explanation | Formula inputs/assumptions/source are viewable |

# 44. What Must Never Happen

LLM invents a pump curve.

LLM invents an operating limit.

Service defaults missing SG to 1.0.

Service assumes every OEM curve is 60 Hz.

Service extrapolates beyond an OEM curve without explicit engineering authorization.

Service silently chooses a PVT/IPR correlation that was not approved.

MCP and REST contain different calculation logic.

A calculation result appears valid when required inputs are missing.

A withdrawn standard is shown as the current compliance authority.

# 45. Current Status from the Four Artifacts

| Area | Current state |
| --- | --- |
| Calculation definitions | Substantially defined across A–G |
| Input schema | Defined, with explicit source/authority and fail-closed rule |
| Reference registry | Defined, with several explicit validation gaps |
| Test cases | Defined: arithmetic + failure modes + real-data placeholders |
| Deterministic execution engine | Not established by these files; this document specifies it |
| OEM master datasets | Required inputs are defined; actual datasets are still dependencies |
| Client-approved limits | Schema exists; actual approved values are dependencies |
| Engineering REST service | Architecture/specification defined here; implementation remains to be built |
| Agent tool integration | Architecture defined here; implementation follows service availability |

# 46. Final Architecture Freeze

LANGGRAPH / AGENT
                                |
                         Engineering Tool
                                |
                           PolicyEngine
                                |
                         +------+------+
                         |             |
                      REST          MCP*
                         |             |
                         +------+------+
                                |
                     ENGINEERING SERVICE
                                |
          +---------------------+----------------------+
          |                     |                      |
    Calculation Registry   Input/Reference       Deterministic
                          Resolution              Calculations
          |                     |                      |
          +---------------------+----------------------+
                                |
                     Result Validation
                                |
                    Provenance + Audit
                                |
                       Engineering Result
                                |
                      +---------+---------+
                      |                   |
                  Evidence Pack        UI/XAI

*MCP is an optional interface to the same Engineering Service logic; it is not a second calculation engine.

# 47. Recommended Immediate Build

Given the current state of the artifacts, the safest implementation path is:

1. Validate/approve the reference-registry gaps.
2. Freeze calculation_registry.yaml version 1.
3. Freeze engineering_input_schema.json version 1.
4. Build the service skeleton + fail-closed executor.
5. Implement A1/A2/A4/A5/C2/F1.
6. Load one real OEM pump curve and implement B1.
7. Load one real motor record and complete B2/C2.
8. Add approved ROR/limit configuration.
9. Expose the service through REST.
10. Add Agent tools.
11. Emit EngineeringResult → Evidence Pack.
12. Show the result in the XAI/chat UI.

# 48. Source Traceability

| Design statement | Primary supplied source |
| --- | --- |
| LLM is not calculation authority / no runtime formula regeneration | ESP_Engineering_Research_Package.md §Purpose/Governing rule; engineering_calculation_registry.yaml lines 9–13 |
| Input sections and fail-closed rule | engineering_input_schema.json lines 782–895 |
| Calculation families/statuses | engineering_calculation_registry.yaml lines 15–406 |
| OEM master-data dependency | engineering_input_schema.json lines 847–857; registry B1–B3 |
| Client limits must not be defaulted | engineering_input_schema.json lines 861–870 |
| Reference validation/withdrawn items | engineering_reference_registry.yaml lines 416–605 |
| Synthetic vs real-data test strategy | engineering_test_cases.yaml lines 615–765 |
| A1–A5 detailed calculation basis | ESP_Engineering_Research_Package.md §2 and engineering_calculation_registry.yaml |

Document conclusion: the supplied artifacts are sufficient to begin the Engineering Service framework and the deterministic MVP calculations, but the full engineering capability remains gated by real OEM datasets, client-approved limits, PVT/well inputs, and resolution of explicitly marked engineering-validation items. The service should be built so blocked calculations remain blocked until those authorities are supplied.